Issue Date; 1/10/2000

Revised: 10/24/2014

Page

Safety Data Sheet

Section 1: Identification

Product Name: Pemrock Common Name: Plaster of Paris Chemical Name: Calcium Sulfate Hemihydrate Manufacturer: Pemaco, Inc. Address: 2030 S. Third St. St. Louis, Mo 63104 Emergency Telephone: (314)231-3399 Fax number : (314)231-4484

Section 2: Hazard(s) Identification

Emergency overview: This product is not expected to produce any unusual hazards beyond nuisance dust during normal use. Effects of overexposure: Acute: Eyes: Direct contact can cause mechanical irritation of eyes. If burning, redness, itching, pain oooooooooooor other symptoms persist or develop, consult physician. Eye irritation Category 2, oooooooooooSubcategory 2B. Skin:This material hardens and slowly becomes hot when mixed with water. Therefore, it ooooooo ooooshould not be used to make a cast enclosing any part of the body. Failure to follow oooooooooooothese instructions can cause sever burns. Inhalation: Exposure to high dust levels may irritate nose, throat, or upper respiratory tract. Ingestion: Plaster is non-toxic, ingestion of a sufficient quantity could lead to obstruction. Chronic:Gypsum displays no specific toxic properties Inhalation: Prolonged and repeated exposure to respirable silica can result in lung disease. Eyes:None Known Skin: None Known Ingestion:Burns to esophagus and stomach

Section 3: Composition/Information on Ingredients

WT Cas number Plaster of Paris (CaSO4-½H2O) >98% 26499-65-0 Silicon Dioxide <1.0 14808-60-7 Page 1 of 4

Section 4: First-Aid Measures

Effects of Acute Over Exposure: Material mixed with water may develop sufficient heat to cause burning of dry skin. Dry particles may cause eye , skin, nose, throat and upper respirator tract irritation. Emergency and First Aid Procedures: Eyes – Flush immediately with water to remove particles, Skin – Flush with water Inhalation – Remove to fresh air Ingestion – Hardens when wetted, may cause obstruction If irritation continues seek medical attention Chronic: Inhalation –Prolonged and repeated exposure to airborne free respirable silica can result in lung disease (i.e., silicosis)and/or lung cancer. The risk of developing silicosis is dependent upon the exposure intensity and duration.

Section 5: Fire-Fighting Measures

Flash Point: Not Combustible Flammable (Explosive) Limit: LEL –NA URL – N/A Special Fire Fighting Procedures and Equipment: N/A Unusual Fire and Explosion Hazards: When heated to decomposition, it will emit toxic fumes of SO2

Section 6: Accidental Release Measures

Steps to be taken in case material is released or spilled: Sweep up, Avoid creating dust. Slurry may clog drains. Waste Disposal Procedures: Normal disposal as for any dry, inert powder

Section 7: Handling and Storage

Handling and Storage: Avoid high humidity and moisture keep bags sealed when not in use..

Section 8: Exposure Controls/Personal Protection

Respiratory Protection: Occupational Safety and Health Administration approved respiration for nuisance dust if above 10 TLV. Ventilation: Local exhaust if above TLV. Eye Protection: Dust goggles or safety glasses Other Personal Protection Equipment: N/A

Section 9: Physical and Chemical Properties

Boiling Point: No Data Available Appearance and odor: Pink or Canary ooooooooooooooooooooooooooooooooooooooooooooooooooooooooooo ppowder, odorless Vapor Pressure(mm Hg): No Data Available Specific Gravity (H2O=1): 2.6-3.2 Vapor Density(air 1 atm): No Data Available % Volatile by Volume(%): 0 Solubility in water: nil Evaporation Rate: N/A. Page 2 of 4

Section 10: Stability and Reactivity

Product Stability: stable in dry environment. Conditions to avoid: Acids Incompatibility (Materials to Avoid): Aluminum(+heat), Diazamethane

Section 11: Toxicological Information

Hazardous decomposition of products: Above 1450°C – SO2 & CaO Hazardous Polymerization: N/A

Section 12: Ecological Information

There is no known causes from this product that would harm the Ecology.

Section 13: Disposal Considerations

Disposal Procedure: Dispose of material in accordance with all applicable federal, state and local regulations. Slurry may clog drains. Do not dispose large quantities directly into waterways.

Section 14: Transport Information

Department of Transportation Not regulated as Hazardous in United States (DOT) Requirements: Canadian Transportation of Nor regulated as dangerous goods Dangerous Goods: UN# None UN Proper Shipping Name: Not Applicable ADNR: None RID/ADR: Not Classified Environmental Hazard: None Annex II of MARPOL 73/78: Not Applicable International Bulk Chemical code: Not Applicable Page 3 of 4

Section 15: Regulatory Information

U.S. EPA’s Toxic Substance Not Listed as reportable quantity in SARA Title III Sections302, 304 Control Act Chemical Substance and313, CAA Section 112(r). Regulated Chemicals for Accidental Inventory: Release Prevention, CERCLA Hazardous Substances, and RCRA 00000000000000000000000000000000 Hazardous Waste’ Canadian Controlled Product Crystalline Silica: IDL* Item #1406 Classification:D2A Regulations: Limestone WHMIS** Classification D2A Portland Cement: WHMIS** Classification: E European Union Directive R36, R37, R38, S37, S38, S39, and S51 67/548/EEC (Annex III and IV): *IDL Item: Canadian Hazardous Product Act Ingredient Disclosure List **WHMIS: Workplace Hazardous Safety Information System

Section 16: Other Information

HMIS: NFPA: Rating Scale: Health – 1 Health - 1 Minimal Hazard =0 Flammability - 0 Flammability - 0 Slight Hazard=1 Physical Hazard – 1 Reactivity – 0 Moderate Hazard = 2 Personal Protection – E Serious Hazard = 3 Extreme Hazard = 4 Revised Date 10/24/2014 New SDS format

Notice: The Information and recommendation contained herein are based on the data believed to be correct as of the date of issue. No guarantee or warranty of any kind, expressed or implied is made with respect to the information contained here in.

Page 4 of 4